# Chapters 20-21 HW (Circuits)

2026-04-03

This HW is due on Test Day, TEST DATE.

Questions and figures are from OpenStax College Physics, 2e (Chapter 20-21) unless otherwise noted.

# Ohm’s Law Problems (Chapter 20)

2

- (P2) A total of 600 C of charge passes through a flashlight in 0.500 h. What is the average current?

- (P18) What current flows through the bulb of a 3.00-V flashlight when its hot resistance is 3.60 Ω?

- (P21) How many volts are supplied to operate an indicator light on a DVD player that has a resistance of 140 Ω, given that 25.0 mA passes through it?

# Circuits Concepts

- (CQ2) Car batteries are rated in ampere-hours ( A h). To what physical quantity do ampere-hours correspond (voltage, charge, current, resistance, etc), and what relationship do ampere-hours have to energy content?

- (Original Question) Is current directly proportional or inversely proportional to voltage? Why does this make sense?

# Equivalent Resistance Problems (Chapter 21)

2

- (P2) (a) What is the resistance of a 100-Ω, a 2.50-kΩ , and a 4.00-kΩ resistor connected in series? (b) In parallel?

- (P3) What are the largest and smallest resistances you can obtain by connecting a 36.0-Ω, a 50.0-Ω, and a 700-Ω resistor together?

- (Original Problem) Find the current going through the battery in the circuit shown in Figure 1.

- Original Figure.

- (Giancoli Ch 19 P16) Determine (a) the equivalent resistance of the circuit shown in Figure 2, (b) thee voltage acros each resistor, and (c) the current through each resistor.

- Giancolli Figure 19-48.

# Equivalent Resistance Concepts

- (Original Question) What happens to resistance when bulbs are wired in series? Why does this make sense?

- (Original Question) What happens to resistance when bulbs are wired in parallel? Why does this make sense?

2

- (P1) (a) What is the resistance of ten 275-Ω resistors connected in series? (b) In parallel?

- (Original Problem) Find the current going through the battery in the circuit shown in Figure 3.

- Original Figure.

# Bonus

- Calculate the equivalent resistance of a cube that has a 1-ohm resistor along each edge, as shown in Figure 4,

- Original Figure. Each resistor has a value of 1 Ω

# Equations

## Electrostatics Equations

F&=kq1q2r2&E&≡Fq=kqr2&V&≡PEq=-Wq

## Circuits Equations

I&≡Δqt&V&=IR&P&=IV=I2R=V2R

Req&=R1+R2+⋯&1Req&=1R1+1R2+⋮

## Unit Conversions

× 103 mC&=1 C& × 106 μC&=1 C& × 109 nC&=1 C

## Constants

| Coulomb Constant | k=9.0 × 109 Nm/C​2 |
| --- | --- |
| Charge of electron/proton | e=±1.60 × 10-19 C |
| Electron mass | me=9.11 × 10-31 kg |
| Proton mass | mp=1.673 × 10-27 kg |
| Neutron mass | mn=1.675 × 10-27 kg |

# Selected Answers

2